Submitted by:

Javeria Rahim (2023-BCS-046)                                           Romaisa (2023-BCS-073)

Malaika (2023-BCS-056)                                                   Sameen Noor (2023-BCS-077)

Submitted to: Sir Majid Shafique

BCS – II (B)

Project Documentation

Interfacing ESP-32 with cable:

Interfacing an ESP32 with a servo motor via cable requires connecting the appropriate pins on the ESP32 to the servo motor and then controlling it with code. Below are detailed steps to achieve this.

### Hardware Connections

- Power Connections:

  - Servo Power (Red Wire): Connect to the 5V pin on the ESP32.

  - Servo Ground (Black/Brown Wire): Connect to the GND pin on the ESP32.

- Signal Connection:

  - Servo Signal (Yellow/Orange Wire): Connect to a GPIO pin on the ESP32, such as GPIO18.

#include <ESP32Servo.h>

// Define the pin where the servo is connected

#define SERVO_PIN 18

// Create a Servo object

Servo myservo;

void setup() {

// Attach the servo on the defined pin to the servo object

myservo.attach(SERVO_PIN);

// Move the servo to the 180 degree position

myservo.write(180);

// Wait for the servo to reach the position

delay(1000);

}

void loop() {

// Move the servo back to 0 degree position

myservo.write(0);

// Wait for the servo to reach the position

delay(1000);

// Move the servo to the 180 degree position

myservo.write(180);

// Wait for the servo to reach the position

delay(1000);

}

Interfacing ESP-32 with Computer:

Interfacing an ESP32 with a computer involves setting up a communication link between the two, typically for the purpose of uploading code, monitoring serial output, or sending/receiving data. Here’s a detailed guide on how to achieve this.

### Required Items

- ESP32 Development Board

- USB Cable (usually micro USB or USB-C depending on your ESP32 board)

- Computer with Arduino IDE installed

### Steps to Interface ESP32 with Computer

#### 1. Install Arduino IDE

Download and install the Arduino IDE from the Arduino website.

#### Install ESP32 Board Support in Arduino IDE

#### 3. Connect ESP32 to the Computer

- Connect the ESP32:

- Use a USB cable to connect your ESP32 development board to your computer.

- Select the ESP32 Board and Port:

- In the Arduino IDE, go to Tools > Board and select your specific ESP32 board model (e.g., ESP32 Dev Module).

- Go to Tools > Port and select the port to which your ESP32 is connected. This is usually labeled as COMx on Windows or /dev/ttyUSBx or /dev/tty.SLAB USB  to UART on macOS/Linux.

#### 4. Upload a Test Sketch

- Open a New Sketch: Go to File > New to open a new sketch.

- Enter the Blink Code:

// Define the pin where the LED is connected

#define LED_PIN 2

// The setup function runs once when the microcontroller is powered on or reset

void setup() {

// Initialize the LED pin as an output

pinMode(LED_PIN, OUTPUT);

}

// The loop function runs over and over again forever

void loop() {

// Turn the LED on (HIGH is the voltage level)

digitalWrite(LED_PIN, HIGH);

// Wait for a second

delay(1000);

// Turn the LED off by making the voltage LOW

digitalWrite(LED_PIN, LOW);

// Wait for a second

delay(1000);

}

- Upload the Sketch:

- Click the Upload button (right-arrow icon) in the Arduino IDE.

- The code will compile and upload to the ESP32. You should see the on-board LED start blinking if the upload is successful.

#### 5. Monitor Serial Output

- Open Serial Monitor:

- Go to Tools > Serial Monitor in the Arduino IDE or press Ctrl+Shift+M.

- Set Baud Rate:

- Ensure the baud rate in the Serial Monitor is set to the same rate as specified in your sketch (usually 115200).

- Modify the Code for Serial Communication:

void setup() {

// Initialize the LED pin as an output

pinMode(LED_PIN, OUTPUT);

// Start the serial communication

Serial.begin(115200);

}

void loop() {

// Turn the LED on (HIGH is the voltage level)

digitalWrite(LED_PIN, HIGH);

// Print a message to the serial monitor

Serial.println("LED is ON");

// Wait for a second

delay(1000);

// Turn the LED off by making the voltage LOW

digitalWrite(LED_PIN, LOW);

// Print a message to the serial monitor

Serial.println("LED is OFF");

// Wait for a second

delay(1000);

}

#### 6. Interfacing for Data Exchange

You can use the serial communication established between the ESP32 and your computer for various purposes such as debugging, data logging, and interactive control.

Code blink for ESP- 32

// Define the pin where the LED is connected

#define LED_PIN 2

// The setup function runs once when the microcontroller is powered on or reset

void setup() {

// Initialize the LED pin as an output

pinMode(LED_PIN, OUTPUT);

}

// The loop function runs over and over again forever

void loop() {

// Turn the LED on (HIGH is the voltage level)

digitalWrite(LED_PIN, HIGH);

// Wait for a second

delay(1000);

// Turn the LED off by making the voltage LOW

digitalWrite(LED_PIN, LOW);

// Wait for a second

delay(1000);

}

Code of the Project:

#include <ESP32Servo.h>

// Define the pin where the servo is connected

#define SERVO_PIN 18

// Create a Servo object

Servo myservo;

void setup() {

// Attach the servo on the defined pin to the servo object

myservo.attach(SERVO_PIN);

// Move the servo to the 180 degree position

myservo.write(180);

}

void loop() {

// Nothing to do in the loop

}